ID515001:

Introduction to Networks

Subnetting Assignment

Learning Outcome

- Design and build a simple local area network using device addressing schemes and basic network configurations.

Student Name:

Course Weighting 20%

Due Date/Time: Friday 27 September

A 10% penalty will be applied for each day late.

Part 1

A) 	Explain the difference between FLSM and VLSM. (3 marks)

B) 	Describe three reasons why it is advantageous to subnet a large network. (3 marks)

C) 	Describe classful addressing. (3 marks)

D) 	Why is classless addressing considered better than classful addressing. (3 marks)

Part 2

Figure 1

A) 	Review Figure 1. You are provided with the network 10.200.2.0 /24.

Using FLSM how many subnet bits are required? How many subnets does this create?

Show working.

(2 marks)

B) 	How many usable hosts are in each subnet created in question A?

Show working.

(2 marks)

C) 	What is the new mask in both dotted decimal and prefix length formats for the subnets created in question A?

(2 marks)

D) 	Provide a table for the FLSM networks in Figure 1. Include the following:

- Network name

- Network IP address

- Broadcast IP address

- First usable IP address

- Last usable IP address

(5 marks)

E) 	Provide a table for the FLSM networks in Figure 1. This time use the starting network of 10.100.128.0 /22. Provide your working for determining the size of each network.

Include the following:

- Network name

- Network IP address

- Broadcast IP address

- First usable IP address

- Last usable IP address

(5 marks)

Part 3

Figure 2

A) 	Review Figure 2. You are provided with the network 10.50.3.0 /24.

You are to use VLSM to create subnets of the correct size to meet the needs of Figure 2. Ensure each network is the minimum size needed.

Provide a table for the VLSM networks in Figure 2. Include the following:

- Network name

- Network IP address

- Broadcast IP address

- First usable IP address

- Last usable IP address

Provide your working for determining the size of each network.

(10 marks)

B)	Using the network information determined in A, provide an additional table that defines a valid IP address for each router interface and each switch management VLAN 1.

Ensure the switch is assigned the last usable IP address in the network.

(5 marks)

Part 4

A) 	What is IPv6 link local addressing used for? (2 marks)

B) 	Explain what SLAAC is and how it works (5 marks)

C) 	Provide an IPv6 addressing scheme in a table for the network topology shown below in figure 3 by subnetting 2001:DB8:ACCA::/48.

Include the following:

IPv6 global unicast address for each router interface.

IPv6 link local address for each router interface.

(5 marks)

Figure 3

Assignment of Marks

You will be assigned marks as follows:-

- Total Marks available	55 marks

| Section | Topic | Mark |
| --- | --- | --- |
| Part 1 | Questions | 12 |
| Part 2 | FLSM | 16 |
| Part 3 | VLSM | 15 |
| Part 4 | IPv6 | 12 |