# Packet Tracer – Skills Integration Challenge (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Addressing Table

| Device | Interface | IP Address / Prefix | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0 | 192.168.0.1 / 25 | N/A |
| R1 | G0/0 | 2001:db8:acad::1/64 | N/A |
| R1 | G0/0 | fe80::1 | N/A |
| R1 | G0/1 | 192.168.0.129 /26 | N/A |
| R1 | G0/1 | 2001:db8:acad:1::1/64 | N/A |
| R1 | G0/1 | fe80::1 | N/A |
| R1 | G0/2 | 192.168.0.193 /27 | N/A |
| R1 | G0/2 | 2001:db8:acad:2::1/64 | N/A |
| R1 | G0/2 | fe80::1 | N/A |
| R1 | S0/0/1 | 172.16.1.2 /30 | N/A |
| R1 | S0/0/1 | 2001:db8:2::1/64 | N/A |
| R1 | S0/0/1 | fe80::1 | N/A |
| Central | S0/0/0 | 209.165.200.226 /30 | N/A |
| Central | S0/0/0 | 2001:db8:1::1/64 | N/A |
| Central | S0/0/0 | fe80::2 | N/A |
| Central | S0/0/1 | 172.16.1.1 /30 | N/A |
| Central | S0/0/1 | 2001:db8:2::2/64 | N/A |
| Central | S0/0/1 | fe80::2 | N/A |
| S1 | VLAN 1 | 192.168.0.2 /25 | 192.168.0.1 |
| S2 | VLAN 1 | 192.168.0.130 /26 | 192.168.0.129 |
| S3 | VLAN 1 | 192.168.0.194 /27 | 192.168.0.193 |
| Staff | NIC | 192.168.0.3 /25 | 192.168.0.1 |
| Staff | NIC | 2001:db8:acad::2/64 | fe80::1 |
| Staff | NIC | fe80::2 | fe80::1 |
| Sales | NIC | 192.168.0.131 /26 | 192.168.0.129 |
| Sales | NIC | 2001:db8:acad:1::2/64 | fe80::1 |
| Sales | NIC | fe80::2 | fe80::1 |
| IT | NIC | 192.168.0.195 /27 | 192.168.0.193 |
| IT | NIC | 2001:db8:acad:2::2/64 | fe80::1 |
| IT | NIC | fe80::2 | fe80::1 |
| Web | NIC | 64.100.0.3 /29 | 64.100.0.1 |
| Web | NIC | 2001:db8:cafe::3/64 | fe80::1 |
| Web | NIC | fe80::2 | Fe80::1 |

# Background / Scenario

The router Central, ISP cluster, and the Web server are completely configured. You must create a new IPv4 addressing scheme that will accommodate 4 subnets using the 192.168.0.0/24 network. The IT department requires 25 hosts. The Sales department needs 50 hosts. The subnet for the rest of the staff requires 100 hosts. A Guest subnet will be added in the future to accommodate 25 hosts. You must also finish the basic security settings and interface configurations on R1. Then, you will configure the SVI interface and basic security settings on switches S1, S2, and S3.

# Instructions

IPv4 Addressing

Use 192.168.0.0/24 to create subnets that meet the host requirements.

Staff: 100 hosts

Sales: 50 hosts

IT: 25 hosts

Guest network to be added later: 25 hosts

Document the IPv4 addresses that have been assigned in the Addressing Table.

Record the subnet for the Guest network:

Type your answers here.

192.168.0.224/27

PC Configurations

Configure the assigned IPv4 address, subnet mask, and default gateway settings on the Staff, Sales, and IT PCs using your addressing scheme.

Assign the IPv6 unicast and link local addresses and default gateways to the Staff, Sales, and IT networks according to the Addressing Table.

R1 Configurations

Configure the device name according to the Addressing Table.

Disable DNS lookup.

Assign Ciscoenpa55 as the encrypted privileged EXEC mode password.

Assign Ciscoconpa55 as the console password and enable login.

Require that a minimum of 10 characters be used for all passwords.

Encrypt all plaintext passwords.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

Configure and enable all the Gigabit Ethernet interfaces.

Configure the IPv4 addresses according to your addressing scheme.

Configure the IPv6 addresses according to the Addressing Table.

Configure SSH on R1:

Set the domain name to CCNA-lab.com

Generate a 1024-bit RSA key.

Configure the VTY lines for SSH access.

Use the local user profiles for authentication.

Create a user Admin1 with a privilege level of 15 and use the encrypted password of Admin1pa55.

Configure the console and VTY lines to log out after five minutes of inactivity.

Block anyone for three minutes who fails to log in after four attempts within a two-minute period.

Switch Configuration

Configure the device name according to the Addressing Table.

Configure the SVI interface with the IPv4 address and subnet mask according your addressing scheme.

Configure the default gateway.

Disable DNS lookup.

Assign Ciscoenpa55 as the encrypted privileged EXEC mode password.

Assign Ciscoconpa55 as the console password and enable login.

Configure the console and VTY lines to log out after five minutes of inactivity.

Encrypt all plaintext passwords.

Connectivity Requirements

Use the web browser on the Staff, Sales, and IT PCs to navigate to www.cisco.pka.

Use the web browser on the Staff, Sales, and IT PCs to navigate to www.cisco6.pka.

All PCs should be able to ping all other the devices.

End of document

# Running Scripts

R1 Configuration

enable

config t

service password-encryption

security passwords min-length 10

hostname R1

login block-for 180 attempts 4 within 120

enable secret 5 $1$mERr$Amm/da5NtiazLuZDbgqZ60

ipv6 unicast-routing

username Admin1 secret 5 $1$mERr$Ty/EkWXcSXEwIckISrps8/

no ip domain-lookup

ip domain-name CCNA-lab.com

interface GigabitEthernet0/0

ip address 192.168.0.1 255.255.255.128

duplex auto

speed auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD::1/64

no shutdown

interface GigabitEthernet0/1

ip address 192.168.0.129 255.255.255.192

duplex auto

speed auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

no shutdown

interface GigabitEthernet0/2

ip address 192.168.0.193 255.255.255.224

duplex auto

speed auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:2::1/64

no shutdown

interface Serial0/0/1

ip address 172.16.1.2 255.255.255.252

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:2::1/64

no shutdown

banner motd ^CAny text^C

line con 0

exec-timeout 5 0

password 7 0802455D0A1606181C1B0D517F

login

line vty 0 4

exec-timeout 5 0

login local

transport input ssh

exit

crypto key generate rsa general-keys modulus 1024

end

S1 Configuration

enable

conf t

service password-encryption

hostname S1

enable secret 5 $1$mERr$Amm/da5NtiazLuZDbgqZ60

no ip domain-lookup

interface Vlan1

ip address 192.168.0.2 255.255.255.128

no shutdown

ip default-gateway 192.168.0.1

line con 0

password 7 0802455D0A1606181C1B0D517F

login

exec-timeout 5 0

line vty 0 4

exec-timeout 5 0

login

line vty 5 15

exec-timeout 5 0

login

end

S2 Configuration

enable

conf t

service password-encryption

hostname S2

enable secret 5 $1$mERr$Amm/da5NtiazLuZDbgqZ60

no ip domain-lookup

interface Vlan1

ip address 192.168.0.130 255.255.255.192

no shutdown

ip default-gateway 192.168.0.129

line con 0

password 7 0802455D0A1606181C1B0D517F

login

exec-timeout 5 0

line vty 0 4

exec-timeout 5 0

login

line vty 5 15

exec-timeout 5 0

login

end

S3 Configuration

enable

conf t

service password-encryption

hostname S3

enable secret 5 $1$mERr$Amm/da5NtiazLuZDbgqZ60

no ip domain-lookup

interface Vlan1

ip address 192.168.0.194 255.255.255.224

no shut

ip default-gateway 192.168.0.193

line con 0

password 7 0802455D0A1606181C1B0D517F

login

exec-timeout 5 0

line vty 0 4

exec-timeout 5 0

login

line vty 5 15

exec-timeout 5 0

login

end