VLSM exercise 2

Subnet 172.16.64.0/18 to create multiple networks based on the following information. The size of each subnet should be the minimum required.

Hint: Assign the largest networks first

Students  – 1200 users

Faculty – 310 users

Admin department – 52 users

For each network record the network address, broadcast address, subnet mask

- First determine how many host bits are needed (to be borrowed) to satisfy each network.

- Start with subnetting the supplied network into subnets large enough to support the largest network.

- Allocate the first of these to the largest network.

- Once created, take the next remaining subnet and determine if it can be allocated appropriately to the next biggest network. If it is too big then subnet that network into smaller ones.

- Continue until a subnet has been created for each required network.

This may help

http://computernetworkingnotes.com/subnetting-supernetting-and-vlsm/vlsm.html

Answer

Students

Our biggest network size is 1200.

Therefore we need 11 host bits (2^11 = 2048) which is the minimum number of host bits to support 1200.

We can’t change the first 18 bits as that was what was given to us.

We therefore allocate the minimum required host bits and mark the remainder as subnet bits.

The octets in binary = 00000000.00000000.01 000 000.00000000

The first 18 bits are the supplied network bits, the next 3 are borrowed bits to create more subnets, the last 11 are the required host bits.

The lowest subnet binary value position = 8 (binary = 01000000). This is the increment for our networks.

We increment until our subnet portion is all 1s 01 111 000 = 120.

Therefore we are splitting (subnetting) the supplied /18 into 8 x /21 networks (2048 ip addresses each).

172.16.64.0 /21 (Students) = 00000000.00000000.01 000 000.00000000

172.16.72.0 /21 = 00000000.00000000.01 001 000.00000000

172.16.80.0 /21 = 00000000.00000000.01 010 000.00000000

172.16.88.0 /21 = 00000000.00000000.01 011 000.00000000

172.16.96.0 /21 = 00000000.00000000.01 100 000.00000000

172.16.104.0 /21 = 00000000.00000000.01 101 000.00000000

172.16.112.0 /21 = 00000000.00000000.01 110 000.00000000

172.16.120.0 /21 = 00000000.00000000.01 111 000.00000000

Allocate the first network to Students.

172.16.64.0/21

Faculty

Split the 2nd /21 (172.16.72.0) network into smaller networks to support Faculty.

Faculty = 310 users = 9 host bits required (2^9 = 512) to make a network supporting 512 ip addresses.

The Octets in binary = 00000000.00000000.01001 00 0.00000000

The first 21 bits are the supplied network bits (Decimal = 72), the next 2 are borrowed bits to create more subnets, the last 9 bits are host bits.

172.16.72.0 /23 (Faculty) = 00000000.00000000.01001 00 0.00000000

172.16.74.0 /23 = 00000000.00000000.01001 01 0.00000000

172.16.76.0 /23 = 00000000.00000000.01001 10 0.00000000

172.16.78.0 /23 = 00000000.00000000.01001 11 0.00000000

Allocate the 1st /23 network to Faculty.

Admin

Split the 2nd /23 network (172.16.74.0) into smaller networks to support Admin.

Faculty = 52 users = 6 host bits required (2^6 = 64) to make a network supporting 64 ip addresses.

The Octets in binary = 00000000.00000000.0100101 0.00 000000

The first 23 bits are the supplied network bits (Decimal = 74), the next 3 are borrowed bits to create more subnets, the last 6 bits are host bits.

172.16.74.0 /26 (Admin)

172.16.74.64 /26

172.16.74.128 /26

172.16.74.192 /26

172.16.75.0 /26

172.16.75.64 /26

172.16.75.128 /26

172.16.75.192 /26

Summary

Usable addresses (not including broadcast and network)

Students	need 2048	172.16.64.1 – 172.16.71.254 /22

Faculty		need 512	172.16.72.1 – 172.16.73.254 /23

Admin		need 64	172.16.74.1 – 172.16.74.62 /26